# Coachline CX-32 Intercity Bus

Operator & Fleet Maintenance Manual

Sample document: this is a fictional vehicle created to demonstrate multi-model retrieval in the Fleet Assistance Copilot. Values are realistic but not taken from any real OEM.

# Document Control

| Field | Value |
| --- | --- |
| Document ID | VM-CX32-OM |
| Vehicle model | CX-32 |
| Variants covered | CX-32 AC Seater (45+1+1), CX-32 AC Sleeper (36 berths) |
| Vehicle category | M3 passenger bus (diesel, BS-VI, 12 m) |
| Region | India |
| Version | 1.0 |
| Effective date | 2026-06-01 |

# 1. About This Manual

This manual covers operation, passenger safety, preventive maintenance and telematics thresholds for the Coachline CX-32 12-metre intercity bus. As a public service vehicle carrying passengers, the CX-32 has stricter behaviour thresholds and additional safety equipment compared with goods vehicles.

# 2. Vehicle Specifications

Table 2.1 – CX-32 technical specifications

| Specification | CX-32 AC Seater | CX-32 AC Sleeper |
| --- | --- | --- |
| Engine | 5.6 L inline-6 diesel, BS-VI, rear-mounted | 5.6 L inline-6 diesel, BS-VI, rear-mounted |
| Maximum power | 250 hp @ 2,300 rpm | 250 hp @ 2,300 rpm |
| Maximum torque | 950 Nm @ 1,200–1,800 rpm | 950 Nm @ 1,200–1,800 rpm |
| Transmission | 6-speed manual (6-speed AMT optional) | 6-speed AMT |
| Seating / berths | 45 passengers + driver + conductor | 36 berths + driver + co-driver |
| Gross vehicle weight (GVW) | 16,200 kg | 16,200 kg |
| Overall length | 12.0 m | 12.0 m |
| Fuel tank capacity | 350 L | 350 L |
| AdBlue (DEF) tank capacity | 40 L | 40 L |
| Speed governor setting | 80 km/h | 80 km/h |
| Brakes | Full air disc brakes with ABS and EBD | Full air disc brakes with ABS and EBD |
| Auxiliary braking | Electromagnetic retarder | Electromagnetic retarder |
| Tyre size | 295/80 R22.5 | 295/80 R22.5 |

# 3. Fluid and Consumable Capacities

Table 3.1 – CX-32 fluid capacities

| System | Capacity | Specification |
| --- | --- | --- |
| Engine oil (with filter) | 22 L | SAE 15W-40, API CK-4 |
| Engine coolant | 55 L | OAT coolant, 50:50 premix (long rear-engine circuit) |
| Transmission oil | 9 L | SAE 75W-90 synthetic |
| Rear axle oil | 13 L | SAE 85W-140, API GL-5 |
| Power steering fluid | 4.5 L | ATF Dexron III |
| Diesel exhaust fluid (AdBlue) | 40 L | ISO 22241 compliant AdBlue |
| Fire suppression agent (FDSS) | 6 L | Aqueous film-forming agent, sealed cylinder |

# 4. Tyre Specifications and Pressures

Table 4.1 – CX-32 tyre pressures

| Position | Cold pressure | Minimum tread depth (company policy) |
| --- | --- | --- |
| Front axle (single) | 7.9 bar (115 psi) | 4.0 mm |
| Rear axle (dual) | 7.6 bar (110 psi) | 3.0 mm |

Retreaded tyres are not permitted on the front axle of any passenger bus. Check the inner dual tyres for pressure matching; a difference of more than 0.3 bar between duals causes uneven wear.

# 5. Passenger Safety Equipment

The driver must confirm during the pre-trip check that all passenger safety equipment below is present, sealed and unobstructed.

Table 5.1 – CX-32 passenger safety equipment

| Equipment | Quantity (Seater) | Quantity (Sleeper) | Location |
| --- | --- | --- | --- |
| Fire extinguisher, 5 kg ABC | 2 | 2 | Driver area and rear |
| Fire detection and suppression system (FDSS) | 1 | 1 | Engine compartment, automatic |
| Emergency exits (rear door + roof hatches) | 1 door + 2 hatches | 1 door + 2 hatches | Rear right, roof |
| Break-glass emergency windows | 4 | 6 | Both sides |
| Emergency hammers | 6 | 8 | Next to break-glass windows |
| AIS-140 emergency (panic) buttons | 4 | 6 | Passenger area, wired to VLTD |
| First-aid kit | 1 | 1 | Driver cabin |
| CCTV cameras | 2 | 3 | Front and saloon |

# 6. Instrument Cluster Warning Lights

Table 6.1 – CX-32 warning lamps

| Warning lamp | Colour | Meaning | Driver action |
| --- | --- | --- | --- |
| Engine compartment fire (FDSS) | Red | Fire detected in engine compartment; suppression discharged | Stop immediately, open all doors, evacuate passengers upwind, call 112. |
| Door open while moving | Red | A passenger door is not fully closed | Stop and close the door; the door-brake interlock prevents moving off with the door open. |
| Low air pressure | Red | System air pressure below 5.5 bar | Stop safely immediately. |
| Engine coolant temperature | Red | Coolant above 110 °C | Stop, idle 2 minutes and switch off; check rear coolant reservoir. |
| Check engine (MIL) | Amber | Engine or emissions fault stored | Book inspection at end of the trip. |
| AdBlue (DEF) low | Amber | DEF level below 10% | Refill AdBlue at the next stop. |
| ABS/EBD fault | Amber | ABS or EBD disabled | Drive gently; book inspection. |
| AC high-pressure fault | Amber | Air-conditioning compressor protection active | Switch AC to fan mode; inform depot. |
| Door-brake interlock engaged | Green | Parking brake held because a door is open | Information only. |

# 7. Common Diagnostic Trouble Codes (DTC)

Table 7.1 – Frequent CX-32 DTCs

| DTC | Description | Severity | Recommended action |
| --- | --- | --- | --- |
| P0217 | Engine over-temperature condition | Critical | Stop safely; check coolant and rear radiator fan. |
| P0128 | Coolant temperature below thermostat regulating temperature | Low | Book thermostat inspection. |
| P0520 | Engine oil pressure sensor circuit fault | Medium | Verify with the mechanical gauge; inspect sensor. |
| P2BAD | NOx exceedence – root cause unknown | Medium | Check AdBlue quality; emissions inspection. |
| B1A50 | FDSS detection circuit fault | High | Fire detection may be disabled; inspect before next trip. |
| C1F10 | Door-brake interlock switch fault | High | Do not operate with passengers until repaired. |

# 8. Preventive Maintenance Schedule

Table 8.1 – CX-32 preventive maintenance schedule

| Interval | Service items |
| --- | --- |
| Daily (pre-trip) | Walk-around, tyres, lights, emergency exits and hammers, fire extinguishers, FDSS indicator, panic buttons, door interlock test |
| Weekly | AC return-air filter cleaning, saloon lighting, CCTV function check |
| Every 10,000 km | Chassis greasing, brake pad wear check, air dryer check |
| 30,000 km or 6 months | Engine oil and oil filter change, fuel filters, air filter inspection |
| 60,000 km or 12 months | AC system service, FDSS inspection and recertification, coolant concentration test, brake disc measurement |
| 120,000 km or 24 months | Coolant replacement, transmission oil replacement, axle oil replacement |

# 9. Telematics Thresholds for CX-32

Passenger buses use the strictest behaviour thresholds in the fleet because standing and sleeping passengers can be injured by forces that would be harmless in a goods vehicle.

Table 9.1 – CX-32 telematics thresholds

| Signal / event | Warning threshold | Critical threshold |
| --- | --- | --- |
| Harsh braking | Deceleration ≥ 0.25 g | Deceleration ≥ 0.35 g |
| Harsh acceleration | Acceleration ≥ 0.20 g | — |
| Harsh cornering | Lateral ≥ 0.22 g | Lateral ≥ 0.30 g |
| Overspeed | > 80 km/h for 10 s | > 85 km/h |
| Door open while moving | — | Any occurrence above 5 km/h |
| Continuous driving | 4 h 30 min without a 30-min break | 5 h without a 30-min break |
| Coolant temperature | ≥ 103 °C | ≥ 110 °C |
| Panic button pressed | — | Any press (immediate control-room alert) |

# 10. Driver Hours and Night Operation

CX-32 night services longer than 8 hours of driving must be operated with two drivers. Drivers must change over at least every 4 hours and each driver must follow the statutory limits in the Fleet Operations Handbook (no more than 5 hours of continuous work without a 30-minute rest). The co-driver must rest in the crew berth, not in a passenger seat.

# 11. Fuel Efficiency Guidance

Table 11.1 – CX-32 expected fuel economy

| Variant / condition | Expected fuel economy | Notes |
| --- | --- | --- |
| AC Seater, highway | 3.8–4.3 km/L | Full passenger load, AC on |
| AC Sleeper, highway | 3.5–4.0 km/L | Higher body weight |
| Any variant, AC off | +10–15% better | AC compressor load removed |

# 12. Emergency Procedures

## 12.1 Engine Compartment Fire

- The FDSS discharges automatically and the red fire lamp comes on.

- Stop at the nearest safe place, apply the parking brake and switch off the engine.

- Open all doors and direct passengers out through the nearest exit, away from traffic and upwind.

- Use the 5 kg extinguishers only if it is safe; never open the engine hatch fully on an active fire.

- Call 112 and the depot control room. Count passengers against the manifest.

## 12.2 Passenger Evacuation

If the main door cannot be used, use the rear emergency door, then roof hatches, then break-glass windows using the emergency hammers. Assist elderly passengers, children and passengers with disabilities first. The conductor or co-driver exits last after checking the saloon and sleeper berths.

## 12.3 Panic Button Alert

When a passenger presses an AIS-140 emergency button, an alert with location is sent to the state emergency response centre and the fleet control room. The driver must not reset the alarm; the control room will call the driver to confirm the situation.

# 13. Frequently Asked Questions

Q: How often is the engine oil changed on the CX-32? A: Every 30,000 km or 6 months, whichever comes first.

Q: Why is the harsh braking threshold lower on the CX-32? A: Passengers can be injured at 0.25 g, so buses are flagged earlier than goods vehicles.

Q: How many panic buttons does the CX-32 Sleeper have? A: Six AIS-140 emergency buttons in the passenger area.