# Hexa HX-400 Heavy Haul Tractor

Operator & Fleet Maintenance Manual

Sample document: this is a fictional vehicle created to demonstrate multi-model retrieval in the Fleet Assistance Copilot. Values are realistic but not taken from any real OEM.

# Document Control

| Field | Value |
| --- | --- |
| Document ID | VM-HX400-OM |
| Vehicle model | HX-400 |
| Variants covered | HX-400 4x2, HX-400 6x4 |
| Vehicle category | N3 heavy goods tractor (diesel, BS-VI) |
| Region | India |
| Version | 1.2 |
| Effective date | 2026-07-01 |

# 1. About This Manual

This manual covers operation, preventive maintenance and telematics thresholds specific to the Hexa HX-400 heavy haul tractor. Where this manual and the Fleet Operations & Driver Safety Handbook differ, the values in this manual take priority for HX-400 vehicles because they are calibrated to this model.

The HX-400 is used for long-haul full-truck-load and container movements. Both variants share the same engine and cab; the 6x4 variant adds a tandem drive axle, a hydraulic retarder and a higher gross combination weight.

# 2. Vehicle Specifications

Key technical specifications for both HX-400 variants are listed below.

Table 2.1 – HX-400 technical specifications

| Specification | HX-400 4x2 | HX-400 6x4 |
| --- | --- | --- |
| Engine | 7.2 L inline-6 diesel, BS-VI | 7.2 L inline-6 diesel, BS-VI |
| Maximum power | 400 hp @ 1,900 rpm | 400 hp @ 1,900 rpm |
| Maximum torque | 1,850 Nm @ 1,000–1,400 rpm | 1,850 Nm @ 1,000–1,400 rpm |
| Transmission | 12-speed AMT | 12-speed AMT |
| Gross combination weight (GCW) | 40,000 kg | 49,000 kg |
| Fuel tank capacity | 415 L | 415 L |
| AdBlue (DEF) tank capacity | 60 L | 60 L |
| Speed governor setting | 80 km/h | 80 km/h |
| Service brakes | Dual-circuit full air, S-cam, ABS | Dual-circuit full air, S-cam, ABS |
| Auxiliary braking | Engine exhaust brake | Engine exhaust brake + hydraulic retarder |
| Tyre size | 295/80 R22.5 | 295/80 R22.5 |

# 3. Fluid and Consumable Capacities

Table 3.1 – HX-400 fluid capacities

| System | Capacity | Specification |
| --- | --- | --- |
| Engine oil (with filter) | 28 L | SAE 15W-40, API CK-4 |
| Engine coolant | 45 L | OAT coolant, 50:50 premix |
| AMT transmission oil | 12 L | SAE 75W-80 synthetic gear oil |
| Rear axle oil (4x2) | 16 L | SAE 85W-140, API GL-5 |
| Rear axle oil (6x4, per axle) | 14 L | SAE 85W-140, API GL-5 |
| Power steering fluid | 5 L | ATF Dexron III |
| Diesel exhaust fluid (AdBlue) | 60 L | ISO 22241 compliant AdBlue |

# 4. Tyre Specifications and Pressures

Check cold tyre pressures before every trip. Hot pressures after a long run are normally 0.5–0.8 bar higher; never bleed air from hot tyres.

Table 4.1 – HX-400 tyre pressures

| Position | Cold pressure | Minimum tread depth (company policy) |
| --- | --- | --- |
| Steer axle | 8.3 bar (120 psi) | 3.0 mm |
| Drive axle(s) | 7.6 bar (110 psi) | 2.0 mm |
| Trailer axles | 8.3 bar (120 psi) | 2.0 mm |

Retreaded tyres must not be fitted on the steer axle of an HX-400. The legal minimum tread depth is 1.6 mm, but company policy requires replacement at the depths above.

# 5. Instrument Cluster Warning Lights

Red lamps mean stop safely as soon as possible. Amber lamps mean the vehicle can continue but must be inspected soon. Green lamps are informational.

Table 5.1 – HX-400 warning lamps

| Warning lamp | Colour | Meaning | Driver action |
| --- | --- | --- | --- |
| Low air pressure | Red | System air pressure below 5.5 bar; buzzer sounds | Stop safely immediately. Do not drive until pressure is above 6.5 bar. |
| Engine coolant temperature | Red | Coolant above 108 °C | Stop, idle 2–3 minutes, then switch off. Do not open the radiator cap when hot. |
| Engine oil pressure | Red | Oil pressure below safe limit | Stop and switch off the engine immediately. |
| Check engine (MIL) | Amber | Engine or emissions fault stored | Continue with care; book inspection within 24 hours. |
| DPF regeneration required | Amber | Soot load high in diesel particulate filter | Perform parked regeneration (30–45 minutes) at a safe location. |
| AdBlue (DEF) low | Amber | DEF level below 10% | Refill AdBlue. Engine torque is derated when the tank is empty. |
| ABS fault | Amber | ABS disabled; base brakes still work | Drive gently, avoid hard braking, book inspection. |
| Water in fuel | Amber | Water detected in fuel pre-filter | Drain the water separator at the next stop. |
| Exhaust brake active | Green | Exhaust brake engaged | Information only. |

# 6. Common Diagnostic Trouble Codes (DTC)

Table 6.1 – Frequent HX-400 DTCs

| DTC | Description | Severity | Recommended action |
| --- | --- | --- | --- |
| P0217 | Engine over-temperature condition | Critical | Stop safely, check coolant level and fan belt; tow to workshop if it recurs. |
| P0524 | Engine oil pressure too low | Critical | Switch off immediately; do not restart until oil level and pressure are checked. |
| P2463 | DPF soot accumulation excessive | High | Run parked regeneration; if it fails, visit workshop within 1 day. |
| P20EE | SCR NOx catalyst efficiency below threshold | Medium | Check AdBlue quality and level; book emissions inspection. |
| P0087 | Fuel rail pressure too low | High | Check fuel level and filters; replace fuel filters if clogged. |
| C0035 | ABS wheel speed sensor circuit fault | Medium | Inspect sensor wiring; ABS may be disabled. |
| U0100 | Lost communication with engine control module | High | Check CAN wiring and telematics unit; inspect at workshop. |

# 7. Preventive Maintenance Schedule

Service at the kilometre or time interval, whichever comes first. For severe duty (mining, ghat sections, more than 20% idling) halve the engine oil interval.

Table 7.1 – HX-400 preventive maintenance schedule

| Interval | Service items |
| --- | --- |
| Daily (pre-trip) | Walk-around, tyre pressure, air tank drain, fluid leaks, lights, fifth-wheel lock, AdBlue level |
| Every 10,000 km | Chassis greasing, fifth-wheel greasing, air dryer check, brake stroke check |
| 40,000 km or 6 months | Engine oil and oil filter change, fuel filters, air filter inspection, DPF differential-pressure check |
| 80,000 km or 12 months | Coolant concentration test, transmission oil level check, brake lining measurement, AdBlue filter replacement |
| 160,000 km or 24 months | Coolant replacement, transmission oil replacement, axle oil replacement, valve clearance adjustment |

# 8. Telematics Thresholds for HX-400

These thresholds are configured in the telematics platform for all HX-400 vehicles. They are tighter than the generic handbook ranges because a laden 40-tonne combination needs much longer stopping distances.

Table 8.1 – HX-400 telematics thresholds

| Signal / event | Warning threshold | Critical threshold |
| --- | --- | --- |
| Harsh braking | Deceleration ≥ 0.30 g | Deceleration ≥ 0.45 g |
| Harsh acceleration | Acceleration ≥ 0.25 g | — |
| Harsh cornering | Lateral ≥ 0.28 g | Lateral ≥ 0.35 g (rollover risk) |
| Overspeed | > 80 km/h for 10 s | > 85 km/h (governor tamper suspected) |
| Excessive idling | > 5 min continuous | > 20% of engine-on time per day |
| Over-revving | > 2,100 rpm | > 2,300 rpm |
| Coolant temperature | ≥ 105 °C | ≥ 108 °C |
| System air pressure | < 6.5 bar | < 5.5 bar |
| AdBlue level | < 10% | 0% (torque derate) |

# 9. Fuel Efficiency Guidance

Keep engine speed in the green band of 1,000–1,500 rpm on the tachometer. The AMT selects gears automatically, but use Eco mode on highways and avoid manual downshifts except on descents.

Table 9.1 – HX-400 expected fuel economy

| Load condition | Expected fuel economy | Notes |
| --- | --- | --- |
| Empty tractor-trailer | 4.2–4.8 km/L | Highway, 60–70 km/h cruise |
| Half load (~20 t GCW) | 3.4–3.8 km/L | Mixed highway |
| Full load (40 t GCW) | 2.6–3.0 km/L | Flat highway; ghat sections lower |

- Use cruise control on flat highways at 60–70 km/h.

- Anticipate traffic and coast in gear instead of braking late.

- Keep tyres at the specified cold pressure; 1 bar under-inflation adds about 2% fuel use.

- Switch off the engine when idling for more than 5 minutes.

# 10. Trailer Coupling and Load Limits

- Apply the trailer parking brake and chock the trailer wheels.

- Check that the fifth-wheel jaws are open and the plate is greased.

- Reverse slowly until the kingpin locks; you will hear a distinct click.

- Perform a tug test in low gear with the trailer brakes applied.

- Visually confirm the lock indicator and safety latch are engaged.

- Connect air lines (red = emergency, yellow = service) and the electrical connector.

- Raise the landing gear fully and remove chocks before moving.

Table 10.1 – HX-400 load limits

| Axle / limit | HX-400 4x2 | HX-400 6x4 |
| --- | --- | --- |
| Front axle load limit | 7,000 kg | 7,500 kg |
| Rear axle load limit | 11,500 kg | 2 × 10,200 kg (tandem) |
| Fifth-wheel load limit | 16,000 kg | 20,000 kg |
| Maximum GCW | 40,000 kg | 49,000 kg |

# 11. Emergency Procedures

## 11.1 Low Air Pressure

If the red low-air lamp and buzzer come on, stop safely at once. Below about 4.5 bar the spring brakes apply automatically and the vehicle will stop on its own. Never try to drive with the spring brakes caged.

## 11.2 Brake Fade on Long Descents

Select a low gear before the descent, keep the exhaust brake (and retarder on 6x4) engaged, and use the service brakes in firm, short applications. If braking feels weak, reduce speed early and use an escape ramp if available.

## 11.3 DPF Regeneration

A parked regeneration takes 30–45 minutes. Park away from dry grass and fuel, keep the engine running and do not switch off during regeneration. Exhaust temperatures exceed 600 °C.

# 12. Frequently Asked Questions

Q: How often should I change the engine oil on the HX-400? A: Every 40,000 km or 6 months, whichever comes first; every 20,000 km on severe duty.

Q: What happens if AdBlue runs out? A: The engine control unit derates torque until AdBlue is refilled. Keep the level above 10%.

Q: At what coolant temperature should I stop? A: Stop when the red coolant lamp comes on at 108 °C; a telematics warning is raised at 105 °C.