# Voltra EV-60 Electric Delivery Van

Operator & Fleet Maintenance Manual

Sample document: this is a fictional vehicle created to demonstrate multi-model retrieval in the Fleet Assistance Copilot. Values are realistic but not taken from any real OEM.

# Document Control

| Field | Value |
| --- | --- |
| Document ID | VM-EV60-OM |
| Vehicle model | EV-60 |
| Variants covered | EV-60 SR (45 kWh), EV-60 LR (60 kWh) |
| Vehicle category | N1 light goods vehicle (battery electric) |
| Region | India |
| Version | 2.0 |
| Effective date | 2026-08-15 |

# 1. About This Manual

This manual covers operation, charging, battery care, preventive maintenance and telematics thresholds for the Voltra EV-60 battery-electric delivery van. The EV-60 has no engine oil, no fuel system and no exhaust after-treatment, so several items in the generic Fleet Operations Handbook (engine oil change, DPF, AdBlue, fuel theft) do not apply to it.

The EV-60 is used for urban and peri-urban last-mile deliveries with daily routes of 80–200 km.

# 2. Vehicle Specifications

Table 2.1 – EV-60 technical specifications

| Specification | EV-60 SR | EV-60 LR |
| --- | --- | --- |
| Battery | 45 kWh LFP | 60 kWh LFP |
| Rated range (MIDC) | 210 km | 280 km |
| Realistic laden city range | 150–170 km | 200–230 km |
| Motor | PMSM, 90 kW peak / 60 kW continuous | PMSM, 90 kW peak / 60 kW continuous |
| Peak torque | 300 Nm | 300 Nm |
| Gross vehicle weight (GVW) | 3,500 kg | 3,500 kg |
| Payload | 1,300 kg | 1,250 kg |
| Cargo volume | 10.5 m³ | 10.5 m³ |
| Top speed | 90 km/h (80 km/h in fleet mode) | 90 km/h (80 km/h in fleet mode) |
| AC charging (on-board charger) | 11 kW, 0–100% in about 5 h | 11 kW, 0–100% in about 6.5 h |
| DC fast charging (CCS2) | 50 kW, 20–80% in about 35 min | 60 kW, 20–80% in about 40 min |
| Tyre size | 215/75 R16C | 215/75 R16C |

# 3. Fluid and Consumable Capacities

Table 3.1 – EV-60 fluid capacities

| System | Capacity | Specification |
| --- | --- | --- |
| Battery and motor coolant loop | 9 L | Low-conductivity EV coolant (OEM spec VTR-C2) |
| Reduction gear oil | 1.8 L | SAE 75W-85 synthetic |
| Brake fluid | 0.9 L | DOT 4 |
| Engine oil | Not applicable | The EV-60 has no engine |
| 12 V auxiliary battery | 70 Ah | AGM type |

# 4. Tyre Specifications and Pressures

Table 4.1 – EV-60 tyre pressures

| Position | Cold pressure (unladen) | Cold pressure (laden) |
| --- | --- | --- |
| Front axle | 4.5 bar (65 psi) | 4.5 bar (65 psi) |
| Rear axle | 4.5 bar (65 psi) | 5.0 bar (73 psi) |

Because the battery makes the EV-60 heavier than a diesel van of the same size, under-inflation reduces range noticeably: 0.5 bar under-inflation reduces range by about 3–4%.

# 5. Battery Care and Charging

The EV-60 uses lithium iron phosphate (LFP) cells, which tolerate full charging. For daily use, charge to 90%. Charge to 100% at least once a week so the battery management system can recalibrate the state-of-charge (SoC) estimate.

Table 5.1 – EV-60 state-of-charge bands

| State of charge (SoC) | Status | What happens / driver action |
| --- | --- | --- |
| 90–100% | Full | Normal. Regenerative braking is reduced above 95%. |
| 20–90% | Normal operating band | Normal driving. |
| 15–20% | Plan charging | Plan a charging stop on the current route. |
| 7–15% | Low SoC warning (amber) | Telematics alert raised; drive directly to a charger. |
| Below 7% | Critical (turtle mode) | Power limited to about 30 kW and top speed to 40 km/h; charge immediately. |

Table 5.2 – EV-60 battery temperature limits

| Battery temperature | Status | Effect |
| --- | --- | --- |
| 15–40 °C | Normal | Full power and full charging speed. |
| 0–15 °C | Cold | DC charging speed reduced; precondition while plugged in. |
| 40–50 °C | Hot | Charging speed reduced to protect cells. |
| ≥ 50 °C | Warning | Amber battery-temperature lamp; power derate begins. |
| ≥ 55 °C | Critical | Red lamp; stop safely and switch off. Contact the EV support desk. |

## 5.1 Charging Procedure

- Park, select P and switch the vehicle off.

- Inspect the charging cable and connector for damage or water; never use a damaged cable.

- Open the charge-port flap and plug in firmly until the connector latches.

- Confirm the charging lamp shows steady green on the charge port.

- To stop, end the session on the charger or app, then unlatch and remove the connector.

- Close the charge-port flap before driving.

Limit DC fast charging to 2 sessions per day where possible. Frequent back-to-back DC fast charging raises battery temperature and accelerates degradation. Battery warranty: 8 years or 160,000 km, or until state of health (SOH) falls below 70%.

# 6. Regenerative Braking

Table 6.1 – EV-60 regenerative braking levels

| Regen level | Deceleration when lifting off | Recommended use |
| --- | --- | --- |
| L0 – Off | Coasting | Long highway descents only when battery is above 95% |
| L1 – Low | Up to 0.08 g | Highway cruising |
| L2 – Medium (default) | Up to 0.14 g | Mixed driving |
| L3 – High (one-pedal) | Up to 0.20 g | Stop-and-go city delivery routes |

Regenerative deceleration up to 0.20 g is not counted as a harsh braking event by the telematics platform.

# 7. Instrument Cluster Warning Lights

Table 7.1 – EV-60 warning lamps

| Warning lamp | Colour | Meaning | Driver action |
| --- | --- | --- | --- |
| EV system fault | Red | High-voltage system fault detected | Stop safely, switch off, do not restart. Call EV support. |
| Battery over-temperature | Red | Battery at or above 55 °C | Stop safely and switch off; keep away from the vehicle if smoke or hissing is present. |
| Low state of charge | Amber | SoC below 15% | Drive directly to a charger. |
| Reduced power (turtle) | Amber | SoC below 7% or thermal derate | Charge immediately; top speed limited to 40 km/h. |
| 12 V charging fault | Amber | DC-DC converter not charging the 12 V battery | Book inspection; the vehicle may not restart. |
| AVAS fault | Amber | Pedestrian warning sound not working | Drive with extra care at low speed; book inspection. |
| READY | Green | High-voltage system active; vehicle can move silently | Information only. Always select P before leaving the seat. |

# 8. Common Diagnostic Trouble Codes (DTC)

Table 8.1 – Frequent EV-60 DTCs

| DTC | Description | Severity | Recommended action |
| --- | --- | --- | --- |
| P0AA6 | High-voltage isolation fault | Critical | Stop; do not touch orange HV cables; tow to an authorised EV workshop. |
| P0A0A | High-voltage interlock circuit open | Critical | Vehicle will not enter READY; tow to EV workshop. |
| P0A7F | Battery pack deterioration | Medium | Schedule a state-of-health test. |
| P0C73 | Motor electronics coolant pump fault | High | Stop when safe; power derate likely; inspect coolant level. |
| U0293 | Lost communication with EV powertrain control module | High | Restart once; if it persists, tow to workshop. |
| P0D2B | On-board charger input voltage out of range | Medium | Try another charger; report the charging location. |

# 9. Preventive Maintenance Schedule

Service at the kilometre or time interval, whichever comes first. There is no engine oil change on the EV-60.

Table 9.1 – EV-60 preventive maintenance schedule

| Interval | Service items |
| --- | --- |
| Daily (pre-trip) | SoC check, tyre visual check, charging cable and port inspection, lights, AVAS sound check |
| 15,000 km or 12 months | Brake inspection, cabin air filter, tyre rotation, 12 V battery test, HV isolation check via diagnostics |
| 30,000 km or 24 months | Brake fluid replacement, coolant level and conductivity check, suspension inspection |
| 60,000 km or 48 months | Reduction gear oil replacement, coolant replacement |
| Every 12 months | Battery state-of-health (SOH) report from the battery management system |

# 10. Telematics Thresholds for EV-60

Electric motors deliver full torque instantly, so harsh acceleration events are more common on the EV-60 than on diesel vans. Thresholds are set accordingly.

Table 10.1 – EV-60 telematics thresholds

| Signal / event | Warning threshold | Critical threshold |
| --- | --- | --- |
| Harsh braking (friction brakes) | Deceleration ≥ 0.40 g | Deceleration ≥ 0.55 g |
| Harsh acceleration | Acceleration ≥ 0.35 g | Acceleration ≥ 0.45 g |
| Harsh cornering | Lateral ≥ 0.35 g | Lateral ≥ 0.45 g |
| Overspeed | > 80 km/h for 10 s | > 90 km/h |
| Energy consumption | > 260 Wh/km (trip average) | > 320 Wh/km |
| State of charge | < 15% | < 7% |
| Battery temperature | ≥ 50 °C | ≥ 55 °C |
| DC fast-charge sessions | > 2 per day | > 4 per day |

# 11. Energy Efficiency Guidance

Table 11.1 – EV-60 expected energy consumption

| Condition | Expected consumption | Notes |
| --- | --- | --- |
| City, unladen | 170–190 Wh/km | Regen L3 |
| City, full payload | 200–230 Wh/km | Regen L3 |
| Highway 70–80 km/h, full payload | 230–260 Wh/km | Aerodynamic drag dominates |
| AC on (any condition) | +8–12% | Set cabin to 24 °C |

- Precondition the cabin while plugged in instead of on battery.

- Use regen level L3 on stop-and-go routes.

- Keep speed at or below 70 km/h on highways; consumption rises sharply above 80 km/h.

- Keep tyres at the laden pressure when carrying more than half payload.

# 12. High-Voltage Safety and Emergency Procedures

## 12.1 High-Voltage Safety

The traction battery operates at about 350 V DC. All high-voltage cables are orange. Drivers must never open covers marked with the high-voltage symbol. Only EV-certified technicians may operate the manual service disconnect.

## 12.2 Accident Involving an EV-60

- Select P, switch the vehicle off and remove the key fob to at least 5 m away.

- Switch on hazard lights and evacuate occupants.

- Tell emergency services that the vehicle is a battery-electric van.

- Do not touch damaged orange cables or leaking fluids.

- If there is smoke, hissing or popping from the battery, move everyone at least 15 m away and upwind.

## 12.3 Battery Fire

A lithium battery fire can re-ignite hours after it appears to be out. Do not attempt to fight a battery fire with a hand-held extinguisher; evacuate and call the fire service. After any battery fire or deep water immersion, the vehicle must be quarantined in the open, at least 15 m from other vehicles and buildings, for 48 hours.

## 12.4 Water Wading

Maximum wading depth is 300 mm at walking pace. Do not drive through water deeper than 300 mm.

# 13. Frequently Asked Questions

Q: Does the EV-60 need an engine oil change? A: No. The EV-60 has no engine; only reduction gear oil is replaced, every 60,000 km or 48 months.

Q: How far should I charge the EV-60 every day? A: Charge to 90% daily and to 100% once a week for SoC calibration.

Q: Why did my EV-60 slow down to 40 km/h? A: Turtle mode activates below 7% SoC or when the battery is too hot; charge the vehicle or let it cool.